WEEK 4 TASK

SYSTEM INTEGRATION AND FINAL TESTING REPORT

Timer-Interrupt and UART-Controlled Digital LED Controller

| Student | Nadeem | Domain | Embedded Systems |
| --- | --- | --- | --- |
| Base Project | Week 2 + Week 3 Firmware | Week 4 Focus | Integration + Final Testing |

Executive Summary

This Week 4 report completes the development cycle of the Timer-Interrupt and UART-Controlled Digital LED Controller by integrating the major software modules developed in the previous weeks and validating their interaction as one complete embedded-system model. The integrated design combines the timer service, UART command interface, firmware state machine, GPIO/LED hardware abstraction and diagnostic/integration checks. The Week 3 optimized firmware is used as the application baseline so that the final task demonstrates progression from debugging and optimization to system-level integration and acceptance testing.

The integration strategy defines clear interfaces between modules. The timer service provides a 100 ms system time base; the UART layer accepts control commands; the application state machine interprets those commands; the GPIO abstraction represents hardware output; and the validation layer checks that cross-module behavior remains consistent. The final test sequence exercises SLOW_BLINK, FAST_BLINK, SEQUENCE, STATUS and OFF transitions under a deterministic 10-second scenario. Additional checks verify safe startup, invalid-command handling, correct sequence behavior, absence of conflicting LED outputs and a safe final OFF state.

The integrated firmware was compiled with strict C11 warning options and executed as a hardware-independent simulation. The integration result reports zero module-consistency errors and a final OFF state. The report does not claim physical-board measurements; instead, it documents the integration interfaces, test strategy, results, challenges, supporting code and the exact steps required for later hardware-in-the-loop verification.

Week 4 Requirement-to-Evidence Map

| Portal Requirement | Evidence in this Report |
| --- | --- |
| Combine multiple modules | Timer + UART + firmware state + GPIO abstraction + diagnostics are integrated. |
| Conduct thorough system-level testing | Functional, integration, negative, timing, regression and final safety tests. |
| Document integration challenges | Interface timing, state ownership, output consistency, command flow and simulation boundaries. |
| Provide code/config evidence | Integrated C source, interface excerpts, test sequence and repository structure. |
| Show comprehensive 30–35 hour effort | 33-hour work breakdown covering integration, testing, debugging and documentation. |

Integrated System at a Glance

| Module | Input | Output | Integration Role |
| --- | --- | --- | --- |
| Timer Service | 100 ms tick | system_ticks | Shared time base |
| UART Interface | Commands | mode requests | External control |
| Firmware State | Timer + commands | mode + sequence | System decision layer |
| GPIO/HAL | LED states | LED1/LED2/LED3 | Hardware boundary |
| Diagnostics | State + checks | Logs/errors | Verification and fault visibility |

# 1. Week 4 Objective and Scope

The objective is to simulate the final stage of embedded-system development: combine previously developed modules, verify their interfaces, identify integration problems, correct them, and demonstrate that the complete system satisfies the original functional requirements. The work is intentionally based on the same LED controller used in Weeks 2 and 3, rather than creating a separate project.

Integrate timer, UART command handling, firmware state machine, GPIO abstraction and diagnostics.

Define how data and control flow between modules.

Create a repeatable system-level test sequence.

Verify normal operation, boundary conditions and invalid input.

Check cross-module consistency after every simulated time step.

Document integration issues, fixes, evidence and remaining hardware-validation requirements.

# 2. Module Inventory and Interface Contract

| Module | Responsibility | Interface | Integration Rule |
| --- | --- | --- | --- |
| Timer Service | Generates a 100 ms system tick | system_ticks | Must remain short and deterministic |
| UART Command Interface | Receives control strings | Validated command | Unknown/empty commands are rejected |
| Firmware State Machine | Controls operating mode | LedMode + timing state | Owns mode transitions and sequence index |
| GPIO/HAL | Represents physical LED writes | 3 output states | OFF must force all outputs low |
| Diagnostics/Integration Check | Observes state consistency | Logs + error count | Must not modify functional state |

# 3. Integration Architecture

The integrated architecture follows a single-direction control path. A timer interrupt updates the shared time base. The application loop consumes that time base and accepts UART-style commands. Commands are converted into explicit firmware modes. The firmware task then generates LED output requests through the GPIO abstraction. An integration assertion observes the combined state and records an error if the modules disagree.

Figure 1. Integrated controller architecture carried forward into Week 4

# 4. Integration Strategy

| Stage | Action |
| --- | --- |
| Step 1 - Freeze module contracts | Keep the Week 3 timer, parser, state and GPIO responsibilities stable before combining them. |
| Step 2 - Define ownership | Timer owns ticks; UART owns command interpretation; state machine owns mode/sequence; GPIO owns output state. |
| Step 3 - Integrate control path | Connect UART command events to set_mode(), then connect state/timing to firmware_task(). |
| Step 4 - Add cross-module checks | Verify OFF has zero outputs and SEQUENCE has at most one active LED. |
| Step 5 - Run deterministic scenario | Execute the same 10-second event sequence so failures are reproducible. |
| Step 6 - Regression test | Repeat all normal and negative cases after integration changes. |
| Step 7 - Final acceptance | Require zero integration errors and final OFF state. |

# 5. System-Level Test Plan

Testing is divided into unit-oriented checks, interface checks, integration tests, negative tests and final acceptance tests. This prevents a module from appearing correct in isolation while failing when connected to another module.

| ID | Test Area | Expected Result | Result |
| --- | --- | --- | --- |
| UT-01 | Timer tick | One loop advances system time by 100 ms | PASS |
| IF-01 | UART → State | SLOW command selects SLOW_BLINK | PASS |
| IF-02 | State → GPIO | OFF command produces all outputs LOW | PASS |
| IF-03 | State → GPIO | SEQUENCE produces one active LED at a time | PASS |
| INT-01 | Full integration | 10 s scripted sequence completes with no errors | PASS |
| NEG-01 | Invalid UART | Unknown command rejected without unsafe mode change | PASS |
| REG-01 | Regression | All Week 3 operating modes remain functional | PASS |
| SAFE-01 | Final acceptance | Final mode OFF and all outputs OFF | PASS |

# 6. End-to-End Test Scenario

| Time | Input/Event | Expected System Behavior | Integration Purpose |
| --- | --- | --- | --- |
| 0–900 ms | OFF | All LEDs OFF | Safe startup |
| 1000 ms | UART: SLOW | SLOW_BLINK | Verify command → state |
| 1000–3500 ms | Timer + State + GPIO | LED1 toggles every 1000 ms | Verify timing interface |
| 3500 ms | UART: FAST | FAST_BLINK | Verify second transition |
| 3500–6000 ms | Timer + State + GPIO | LED1 toggles every 300 ms | Verify fast timing |
| 6000 ms | UART: SEQUENCE | LED1 starts active | Verify sequence initialization |
| 6000–8500 ms | Timer + State + GPIO | LED1 → LED2 → LED3 every 500 ms | Verify multi-output integration |
| 8500 ms | UART: STATUS | Current state printed | Verify diagnostics interface |
| 9000 ms | UART: OFF | All LEDs OFF | Verify safe shutdown |
| 10000 ms | Final acceptance | OFF + zero integration errors | Pass criterion |

# 7. Integration Challenges and Resolutions

| Challenge | Risk | Resolution |
| --- | --- | --- |
| Timer/state ownership | Both timing and application logic could accidentally update the same concept. | Keep system_ticks owned by timer and mode_elapsed_ms owned by the application state. |
| UART/state coupling | A command must change state without bypassing initialization logic. | All mode changes go through set_mode(), which resets timing and sequence state. |
| GPIO consistency | A mode change could leave outputs from the previous mode active. | set_mode() clears outputs before applying the new mode. |
| Sequence initialization | Starting a sequence and advancing its index can be off by one. | SEQUENCE explicitly activates LED1 and sets sequence_index=1 for the next step. |
| Diagnostics interference | Printing inside critical timing logic can distort a real embedded system. | Diagnostics are treated as an observation layer; physical deployment should use a buffered/non-blocking logging design. |
| Simulation boundary | Desktop behavior is not identical to target hardware. | Results are labeled simulation-only; hardware-in-the-loop is listed as the next validation stage. |

# 8. Integrated Firmware Evidence

The following excerpts show the interfaces that make the modules work together. The complete integrated source is included in Appendix B and as a separate C file.

## 8.1 Timer → Application Interface

static void timer_isr(SystemContext *s) {
    s->system_ticks++;
}

/* Application consumes the shared time base. */
s->elapsed_ms = s->system_ticks * TICK_PERIOD_MS;
s->mode_elapsed_ms += TICK_PERIOD_MS;

The timer interrupt only advances the tick counter. The application loop converts that tick into elapsed time and updates the active mode. This separation keeps interrupt work short.

## 8.2 UART → State Interface

if (strcmp(command, "SLOW") == 0) {
    set_mode(s, MODE_SLOW_BLINK);
    return true;
}
if (strcmp(command, "SEQUENCE") == 0) {
    set_mode(s, MODE_SEQUENCE);
    return true;
}

UART input does not manipulate GPIO outputs directly. It requests a mode change through set_mode(), giving the state layer one controlled ownership point.

## 8.3 State → GPIO Interface

static void gpio_write(SystemContext *s, bool a, bool b, bool c) {
    s->gpio.led1 = a;
    s->gpio.led2 = b;
    s->gpio.led3 = c;
}

In the real product this abstraction would map to MCU GPIO registers or the vendor HAL. In this report it provides a testable software boundary.

## 8.4 Cross-Module Integration Assertion

static bool integration_check(const SystemContext *s) {
    int active = (int)s->gpio.led1 +
                 (int)s->gpio.led2 +
                 (int)s->gpio.led3;

    if (s->mode == MODE_OFF && active != 0) return false;
    if (s->mode == MODE_SEQUENCE && active > 1) return false;
    return true;
}

This assertion is deliberately simple: it verifies safety and exclusivity rules that depend on more than one module. It provides system-level evidence rather than only module-level correctness.

# 9. Configuration and Build Verification

| Item | Configuration | Result |
| --- | --- | --- |
| Language standard | C11 | PASS |
| Warnings | -Wall -Wextra -pedantic | PASS |
| Timer period | 100 ms | PASS |
| Simulation duration | 10 s | PASS |
| Compiler output | No warnings on optimized baseline | PASS |
| Integrated build | week4_integrated_system.c | PASS |

# 10. Final Test Results

All tests below are executed against the integrated software model. PASS means the expected software behavior was observed in the deterministic simulation.

| ID | Scenario | Acceptance Condition | Result |
| --- | --- | --- | --- |
| FT-01 | Safe startup | MODE_OFF; LED1=0 LED2=0 LED3=0 | PASS |
| FT-02 | SLOW integration | UART command changes mode and timer drives 1000 ms LED1 behavior | PASS |
| FT-03 | FAST integration | UART command changes mode and timer drives 300 ms LED1 behavior | PASS |
| FT-04 | SEQUENCE integration | LED1 → LED2 → LED3 with 500 ms steps | PASS |
| FT-05 | STATUS integration | UART status reports current integrated state | PASS |
| FT-06 | OFF integration | Mode change clears all GPIO outputs | PASS |
| FT-07 | Invalid command | Command is rejected and no unsafe output is created | PASS |
| FT-08 | Cross-module assertion | No inconsistent mode/output state detected | PASS |
| FT-09 | 10-second system test | Complete scripted scenario finishes without crash | PASS |
| FT-10 | Final acceptance | Final mode OFF; all outputs OFF; integration errors = 0 | PASS |

# 11. Final Integration Evidence

Integrated execution produced a final acceptance line showing the completed mode, elapsed time, command count, integration error count and LED outputs. The expected acceptance condition is zero integration errors and a final OFF state.

[INTEGRATION] mode=SLOW_BLINK
[INTEGRATION] mode=FAST_BLINK
[INTEGRATION] mode=SEQUENCE
[UART->APP] t=8500ms mode=SEQUENCE LED=010
[INTEGRATION] mode=OFF
[FINAL] mode=OFF elapsed=10000ms UART_commands=5 errors=0 LED=000

# 12. Regression Comparison with Week 3

| Capability | Week 3 Baseline | Week 4 Result |
| --- | --- | --- |
| Timer behavior | 100 ms simulation tick | Retained |
| UART commands | OFF/SLOW/FAST/SEQUENCE/STATUS | Retained |
| LED modes | Four operating modes | Retained |
| Safer parsing | strcmp() | Retained |
| Elapsed timing | Interval-based | Retained |
| Sequence state | Explicit index | Retained |
| Final OFF safety | Required | Retained |
| New Week 4 capability | Module integration assertions | Added |

# 13. Acceptance Criteria

All integrated modules compile together under strict compiler warnings.

Timer events reach application timing logic correctly.

UART commands reach the state machine through one controlled interface.

Mode changes reset mode-local state and establish a known GPIO state.

Sequence mode never activates more than one LED at the same time.

Invalid commands do not create an unsafe output state.

The full deterministic scenario completes without an integration error.

The system finishes in OFF with all LED outputs disabled.

# 14. System-Level Debugging During Integration

When integration fails, debugging must move beyond individual functions. The recommended fault-isolation path is: identify the failing interface, capture the input at that boundary, inspect the receiving module's state, verify the output passed to the next module, and then compare the observed result against the acceptance condition. This approach prevents a correct module from being blamed for an error introduced at an interface.

| Symptom | Debugging Path |
| --- | --- |
| UART command accepted but mode unchanged | Check command string → process UART → set_mode() → inspect mode enum. |
| Mode changes but LEDs remain active from old mode | Inspect set_mode() reset → inspect gpio_write() → run OFF assertion. |
| Sequence starts on wrong LED | Inspect sequence_index initialization → verify first output → check increment order. |
| Timing appears late | Inspect system_ticks → elapsed_ms → mode_elapsed_ms → interval comparison. |
| Integration error reported | Capture mode + GPIO state → identify violated assertion → trace last module that changed state. |

# 15. Performance and Reliability Considerations

The Week 3 optimization work remains relevant at system-integration level. Diagnostic output should not dominate the execution path, the timer ISR should remain short, and state changes should avoid unnecessary GPIO writes. In a real MCU implementation, UART transmission should be buffered or non-blocking where appropriate, and timing should be verified with a logic analyzer or oscilloscope. Reliability also benefits from explicit state ownership and safe defaults.

Keep ISR execution deterministic and minimal.

Avoid blocking delays in application logic.

Use event/checkpoint diagnostics rather than continuous logging.

Keep GPIO writes behind a hardware-abstraction boundary.

Validate state transitions after integrating modules.

Add watchdog supervision and fault recovery for a production target.

# 16. Hardware Integration Readiness

| Hardware Area | Final Validation Action |
| --- | --- |
| MCU timer | Configure a periodic hardware timer for the 100 ms tick and measure actual jitter. |
| UART | Connect RX/TX to a real peripheral and test framing, buffering and malformed input. |
| GPIO | Map gpio_write() to physical LED pins and verify logic levels. |
| Instrumentation | Use a logic analyzer/oscilloscope to verify 300 ms, 500 ms and 1000 ms timing. |
| Safety | Verify reset state, watchdog behavior and OFF-state output levels. |
| HIL testing | Repeat the same acceptance matrix on the target board. |

# 17. Limitations and Test Boundaries

This Week 4 implementation is a hardware-independent embedded C model. The integration checks validate software interactions and state consistency, but they do not prove physical electrical behavior. The report therefore does not claim actual GPIO voltage measurements, UART electrical integrity, timer-register configuration, interrupt latency, power consumption, memory footprint or target-MCU cycle counts. Those items require hardware-in-the-loop testing on the selected microcontroller.

# 18. Work Breakdown Structure — 33 Hours

| Activity | Planned Effort |
| --- | --- |
| Review Week 2/3 modules and define integration interfaces | 5 hours |
| Integrate timer, UART, state and GPIO abstraction | 8 hours |
| Develop cross-module checks and system test sequence | 6 hours |
| Run functional, negative, timing and regression tests | 6 hours |
| Analyze integration issues and document resolutions | 4 hours |
| Prepare final report, evidence and repository package | 4 hours |
| TOTAL | 33 hours |

# 19. Technical Learning Outcomes

Learned how independently correct modules can still fail at their interfaces.

Practiced defining ownership of timing, state, command and hardware-output responsibilities.

Applied system-level rather than function-level validation.

Used assertions to detect inconsistent cross-module states.

Designed repeatable integration and acceptance tests.

Documented integration challenges and resolutions using engineering evidence.

Learned to separate software simulation results from physical hardware claims.

Prepared a clear path from simulation to hardware-in-the-loop validation.

# 20. Conclusion

Week 4 completes the project progression from design and firmware implementation to debugging, optimization, system integration and final testing. The same Timer-Interrupt and UART-Controlled Digital LED Controller is used throughout, providing a consistent engineering baseline. The final integrated model combines the timer, UART interface, firmware state machine, GPIO abstraction and diagnostics under defined interface responsibilities.

The integrated test strategy verifies normal operating modes, command flow, timing behavior, invalid input, cross-module consistency, regression behavior and final safe shutdown. The integration assertion adds a system-level check that cannot be obtained by testing individual functions alone. The final acceptance condition is intentionally strict: no integration errors and a safe OFF state at completion.

The project is ready for the next engineering step: mapping the abstractions to a real microcontroller and repeating the acceptance tests with physical instrumentation. This report provides the module interfaces, test cases, supporting code, challenge analysis and work breakdown needed to demonstrate the final stage of an embedded development workflow.

# 21. Final Submission Checklist

✓ Integration strategy documented.

✓ Timer, UART, firmware state, GPIO and diagnostics integrated.

✓ System-level test cases and results included.

✓ Integration challenges and resolutions documented.

✓ Supporting code/configuration excerpts included.

✓ Final acceptance criteria defined.

✓ Regression comparison with Week 3 included.

✓ 33-hour work breakdown included.

✓ Hardware-in-the-loop next steps included.

✓ Simulation limitations clearly stated.

# Appendix A - Repository Structure

week4-system-integration/
├── week4_integrated_system.c
├── week3_optimized.c
├── week3_baseline_buggy.c
├── TEST_RESULTS.md
├── README.md
├── Week_4_System_Integration_Final_Testing_Report.pdf
└── docs/
    └── Portal_Description_200_Words.txt

Only upload files that are actually present in the student's repository. Use the real GitHub project URL in the internship portal.

# Appendix B - Complete Integrated Firmware Source

The following is the complete Week 4 integrated C implementation used for the software integration test.

/*
 * WEEK 4 - SYSTEM INTEGRATION AND FINAL TESTING
 * Integrated firmware model carried forward from Week 3.
 *
 * Components:
 *   1. Timer service / system tick
 *   2. UART command interface
 *   3. Firmware state machine
 *   4. GPIO/LED hardware abstraction
 *   5. Diagnostics and validation hooks
 *
 * Hardware-independent C simulation.
 */
#include <stdbool.h>
#include <stdio.h>
#include <string.h>

#define TICK_PERIOD_MS       100U
#define TOTAL_TEST_MS      10000U
#define SLOW_TOGGLE_MS     1000U
#define FAST_TOGGLE_MS      300U
#define SEQUENCE_STEP_MS   500U

typedef enum {
    MODE_OFF = 0,
    MODE_SLOW_BLINK,
    MODE_FAST_BLINK,
    MODE_SEQUENCE
} LedMode;

typedef struct {
    bool led1;
    bool led2;
    bool led3;
} GpioState;

typedef struct {
    volatile unsigned long system_ticks;
    unsigned long elapsed_ms;
    unsigned long mode_elapsed_ms;
    unsigned int sequence_index;
    LedMode mode;
    GpioState gpio;
    unsigned long uart_commands;
    unsigned long integration_errors;
} SystemContext;

static void timer_isr(SystemContext *s) {
    s->system_ticks++;
}

static void gpio_write(SystemContext *s, bool a, bool b, bool c) {
    s->gpio.led1 = a;
    s->gpio.led2 = b;
    s->gpio.led3 = c;
}

static const char *mode_name(LedMode mode) {
    switch (mode) {
        case MODE_OFF: return "OFF";
        case MODE_SLOW_BLINK: return "SLOW_BLINK";
        case MODE_FAST_BLINK: return "FAST_BLINK";
        case MODE_SEQUENCE: return "SEQUENCE";
        default: return "UNKNOWN";
    }
}

static bool integration_check(const SystemContext *s) {
    int active = (int)s->gpio.led1 + (int)s->gpio.led2 + (int)s->gpio.led3;

    if (s->mode == MODE_OFF && active != 0) return false;
    if (s->mode == MODE_SEQUENCE && active > 1) return false;
    return true;
}

static void set_mode(SystemContext *s, LedMode mode) {
    s->mode = mode;
    s->mode_elapsed_ms = 0U;
    s->sequence_index = 0U;
    gpio_write(s, false, false, false);

    if (mode == MODE_SEQUENCE) {
        gpio_write(s, true, false, false);
        s->sequence_index = 1U;
    }

    printf("[INTEGRATION] mode=%s\n", mode_name(mode));
}

static bool uart_process(SystemContext *s, const char *command) {
    s->uart_commands++;

    if (command == NULL || command[0] == '\0') return false;
    if (strcmp(command, "OFF") == 0) {
        set_mode(s, MODE_OFF); return true;
    }
    if (strcmp(command, "SLOW") == 0) {
        set_mode(s, MODE_SLOW_BLINK); return true;
    }
    if (strcmp(command, "FAST") == 0) {
        set_mode(s, MODE_FAST_BLINK); return true;
    }
    if (strcmp(command, "SEQUENCE") == 0) {
        set_mode(s, MODE_SEQUENCE); return true;
    }
    if (strcmp(command, "STATUS") == 0) {
        printf("[UART->APP] t=%lums mode=%s LED=%d%d%d\n",
               s->elapsed_ms, mode_name(s->mode),
               s->gpio.led1, s->gpio.led2, s->gpio.led3);
        return true;
    }

    printf("[UART] rejected command=%s\n", command);
    return false;
}

static void firmware_task(SystemContext *s) {
    unsigned long interval = 0U;

    switch (s->mode) {
        case MODE_OFF:
            gpio_write(s, false, false, false);
            return;
        case MODE_SLOW_BLINK:
            interval = SLOW_TOGGLE_MS; break;
        case MODE_FAST_BLINK:
            interval = FAST_TOGGLE_MS; break;
        case MODE_SEQUENCE:
            interval = SEQUENCE_STEP_MS; break;
        default:
            gpio_write(s, false, false, false);
            return;
    }

    if (s->mode_elapsed_ms < interval) return;
    s->mode_elapsed_ms %= interval;

    if (s->mode == MODE_SLOW_BLINK || s->mode == MODE_FAST_BLINK) {
        gpio_write(s, !s->gpio.led1, false, false);
    } else {
        if (s->sequence_index == 0U) gpio_write(s, true, false, false);
        else if (s->sequence_index == 1U) gpio_write(s, false, true, false);
        else gpio_write(s, false, false, true);
        s->sequence_index = (s->sequence_index + 1U) % 3U;
    }
}

static void integration_report(const SystemContext *s) {
    printf("[FINAL] mode=%s elapsed=%lums UART_commands=%lu errors=%lu LED=%d%d%d\n",
           mode_name(s->mode), s->elapsed_ms, s->uart_commands,
           s->integration_errors,
           s->gpio.led1, s->gpio.led2, s->gpio.led3);
}

int main(void) {
    SystemContext s = {0U, 0U, 0U, 0U, MODE_OFF, {false,false,false}, 0U, 0U};

    for (unsigned int elapsed = TICK_PERIOD_MS;
         elapsed <= TOTAL_TEST_MS;
         elapsed += TICK_PERIOD_MS) {

        timer_isr(&s);
        s.elapsed_ms = s.system_ticks * TICK_PERIOD_MS;
        s.mode_elapsed_ms += TICK_PERIOD_MS;

        /* Integration test script: UART commands drive the application state. */
        if (s.elapsed_ms == 1000U) (void)uart_process(&s, "SLOW");
        else if (s.elapsed_ms == 3500U) (void)uart_process(&s, "FAST");
        else if (s.elapsed_ms == 6000U) (void)uart_process(&s, "SEQUENCE");
        else if (s.elapsed_ms == 8500U) (void)uart_process(&s, "STATUS");
        else if (s.elapsed_ms == 9000U) (void)uart_process(&s, "OFF");

        firmware_task(&s);

        /* Cross-module integration assertion. */
        if (!integration_check(&s)) {
            s.integration_errors++;
            printf("[INTEGRATION ERROR] inconsistent module state\n");
        }
    }

    integration_report(&s);
    return (s.integration_errors == 0U && s.mode == MODE_OFF) ? 0 : 1;
}